Comms Data Interchange Format

Specification Version 1.1

IQGeo, NINE Hills Road, Cambridge, CB2 IGE, United Kingdom. 
Telephone: +44 (0) 1223 606655. Website: https://www.iqgeo.com/.

Version History

| CDIF Version | Product Version | Comment |
| --- | --- | --- |
| 1.0 | 2.1 | Initial version |
| 1.1 | 3.0 | Add support for line of count, cable offset geometry and cable segment association to equipment. |

Contents

1	Overview	4

1.1	Package Structure	4

1.2	File Format	4

1.3	Feature Categories	5

1.4	Feature Identifiers	5

2	Feature Data	6

2.1	Structure	6

2.2	Route	6

2.3	Equipment	6

2.4	Conduit	7

2.5	Tube	7

2.6	Tube Segment	7

2.7	Cable	8

2.8	Cable Slack	8

2.9	Cable Segment	9

2.10	Connection	9

2.11	Line of Count	10

2.12	Line of Count Section	10

3	Field MetaData	12

4	Package Metadata	13

# Overview

CDIF is a IQGeo format for exchanging telecom network data between database systems, retaining connectivity and containment relationships.

## Package Structure

A CDIF package consists of a set of comma-separated variable (CSV) files. It is usually supplied in zipped format. A typical structure is shown below.

Each package file contains one of the following types of data:

| File Type | Suffix | Description | Optional |
| --- | --- | --- | --- |
| Feature data | .csv | ID, location and attributes of instances of a feature type | No |
| Field metadata | .fields | Describes attributes of a feature type (data type, units) | Yes |
| Package metadata | .metadata | Global package properties (version, coordinate system, ...) | Yes |

By convention, feature and field metadata files are grouped into sub-directories based on the category to which they belong. However, any file grouping (or none) is permitted.

## File Format

All files have the following properties:

- Comma-separated variable text (CSV) with headings row

- Character encoding UTF8

- Geometry values represented as EWKB (extended well-known binary format)

- Boolean values represented as true or false

- Date values represented as %Y-%m-%d

- Timestamp files represented as %Y-%m-%dT%H:%M:%S.%f

- Null value indicated by empty string

For feature and field metadata files the first component of the file name defines the feature type to which the file relates e.g. pole.1.csv contains features of type pole.

The coordinate system in which geometries are represented can be specified in the package metadata.  If not provided it is assumed to be WGS84 long/lat decimal degrees.  Note that a feature can have more than one geometry field.

## Feature Categories

The attributes provided for a feature type depend on the category to which it belongs.  Categories are as follows:

| Category | Description |
| --- | --- |
| Structure | A building, manhole, pole or other top-level point feature. |
| Route | A path between two structures (a trench, aerial route or similar). |
| Equipment | A signal carrying device (splitter, mux, ...) or enclosure (splice closure, rack, room, ...) within a structure |
| Conduit | A simple duct running between two structures |
| Tube | A duct that can potentially pass unbroken through structures |
| Tube Segment | A section of a tube between two structures |
| Cable | A pyhsical cable within the network |
| Cable Slack | A coil of cable within a structure |
| Cable Segment | A section of a cable between two structures (or within a structure) |
| Connection | A fiber-level connection from one range of fibers (or ports) to another range of fibers (or ports) |
| Line of Count | Assignment of name and status to strands in the network. |

## Feature Identifiers

Each feature record has a ID that is globally unique within the package. This allows it to be referenced unambiguously in connectivity and containment relationships.

By convention, feature identifiers usually have the form:

<feature_type>/<row_no>      e.g.   pole/262

However, any string or number can be used provided it is unique within the package.

# Feature Data

This section describes the table structure for each type of feature data that can occur in the package.

## Structure

A structure models a building, manhole, pole or other top-level point feature.

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| location | point | No | Location in geographic world |
| myw_orientation_location | float | Yes | Symbol orientation (degrees clockwise from north) |
| <feature-specific field> | ... | Yes | Feature type-specific attribute |
| ... | ... | ... | ... |

Notes:

- Feature-specific fields will vary by structure type (height of pole, specification of manhole, ...)

- The package will contain a separate file for each structure type present in the data.

## Route

A route models an direct path between two structures (a trench, aerial route or similar).

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| in_structure | feature_id | No | Start structure (if any) |
| out_structure | feature_id | No | End structure (if any) |
| path | linestring | No | Path in geographic world (from start to end) |
| length | double | Yes | Measured length (in m) |
| <feature-specific field> | ... | Yes | Feature type-specific attribute |
| ... | ... | ... |  |

Notes:

- Feature-specific fields will vary by route type (depth of trench, owner of strand, ...)

- The package will contain a separate file for each route type present in the data.

## Equipment

An equipment feature models a signal carrying device (splitter, mux, ...) or enclosure (splice closure, rack, room, ...).

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| n_<tech>_in_ports | integer | Yes | Number of fiber input ports (if any). <tech> is one of fiber, copper or coax |
| n_<tech>_out_ports | integer | Yes | Number of fiber output ports (if any). <tech> is one of fiber, copper or coax. |
| housing | feature_id | No | Equipment or structure that is direct container |
| root_housing | feature_id | Yes | Structure in which self is housed |
| location | point | Yes | Location in geographic world |
| offset_geom | Point | Yes | Offset Location (for coax equipment) |
| <feature-specific field> | ... | Yes | Feature type-specific attribute |
| ... | ... |  | ... |

Notes:

- If root_housing and location are supplied their values must match those of housing

- Feature-specific fields vary by equipment type type (asset ID, room number, ...)

- The package will contain a separate file for each equipment type present in the data.

## Conduit

A conduit models models an duct running between two structures. Conduits can be contained directly within a route or nested within other conduits.

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| housing | feature_id | No | Conduit or route that is direct container |
| root_housing | feature_id | Yes | Route in which rus |
| in_structure | feature_id | Yes | Start structure |
| out_structure | feature_id | Yes | End structure |
| path | linestring | Yes | Path in geographic world from start to end |
| <feature-specific field> | ... | Yes | Feature type-specific attribute |
| ... | ... | ... | ... |

Notes:

- If root_housing, in_structure, out_structure and path are supplied their values must match those of housing

- Feature-specific fields vary by conduit type

- The package will contain a separate file for each conduit type present in the data.

## Tube

A tube models a duct that can potentially pass unbroken through structures e.g. a blown fiber tube. Internally it consists of a chain of tube segments.

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| path | linestring | No | Path in geographic world from start to end |

Notes:

- The file will be present if there are any continuous tubes in the data.

## Tube Segment

A tube segment models a section of a tube between two structures.

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| in_conduit | feature_id | No | Previous conduit in run (if any) |
| out_conduit | feature_id | No | Next conduit in run (if any) |
| conduit_run | feature_id | No | Tube of which self is a part |
| housing | feature_id | No | Conduit, tube segment or route that is direct container |
| root_housing | feature_id | Yes | Route in which segment runs |
| in_structure | feature_id | Yes | Start structure |
| out_structure | feature_id | Yes | End structure |
| path | linestring | Yes | Path in geographic world from start to end |
| <feature-specific field> | ... | Yes | Feature type-specific attribute |
| ... | ... | ... | ... |

Notes:

- If root_housing, in_structure, out_structure and path are supplied their values must match those of housing

- Feature-specific fields will vary by tube type

- The package will contain a separate file for each conduit type present in the data.

## Cable

A cable models a pyhsical cable within the network. Internally it consists of a chain of cable segments. Connections can be made at any structure along its path (not just at its end points).

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| <tech>_count | integer | Yes | Number of fibers, copper pairs or coax wires. Tech is one of fiber, copper or coax |
| directed | boolean | No | True if direction is meaningful to user |
| path | linestring | No | Path in geographic world (from start to end) |
| placement_path | linestring | Yes | Path entered by user to set route (if routed manually) |
| offset_geom | linestring | Yes | Geoschematic path |
| <feature-specific field> | ... | Yes | Feature type-specific attribute |
| ... | ... | ... | ... |

Notes:

- Feature-specific fields will vary by cable type

- The package will contain a separate file for each cable type present in the data.

## Cable Slack

A cable slack models a coil of cable within a structure.

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| cable | feature_id | No | Cable of which self is a part |
| length | double | No | Length (in m) |
| housing | feature_id | No | Equipment or structure that is direct container |
| root_housing | feature_id | Yes | Structure in which self is housed |
| location | point | Yes | Location in geographic world |
| <feature-specific field> | ... | Yes | Feature type-specific attribute |
| ... | ... |  | ... |

Notes:

- Every slack record must have a corresponding cable segment record. That segment must name the slack as its housing and have a length matching the slack record length

- If location is supplied it must match that of housing

- Feature-specific fields will vary by cable type

- The package will contain a separate file for each cable type present in the data.

## Cable Segment

A cable segment models a section of a cable between two structures (or within a structure).

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| cable | feature_id | No | Cable of which self is a part |
| directed | boolean | Yes | Must match cable.directed |
| length | double | Yes | Measured length (in m) |
| forward | boolean | No | True if self path runs in same direction as root_housing |
| in_segment | feature_id | No | Previous segment in chain (if any) |
| out_segment | feature_id | No | Next segment in chain (if any) |
| housing | feature_id | No | Conduit, tube segement, route, equipment or structure that is self's direct container |
| root_housing | feature_id | Yes | Route or structure in which self runs |
| in_structure | feature_id | Yes | Start structure |
| out_structure | feature_id | Yes | End structure |
| In_equipment | feature_id | Yes | Start equipment (if explicitly contained) |
| out_equipment | feature_id | Yes | End equipment (if explicitly contained) |
| path | linestring | Yes | Path in geographic world from start to end |

Notes:

- If root_housing, in_structure, out_structure and path are supplied they must match those of housing (adjusted for forward flag, if necessary).

- For an internal segment, in_structure, out_structure and root_housing will have the same value.

- Fields in_equipment and out_equipment are only populated where containment cannot be inferred from fiber-level connectivity.

## Connection

A connection models a fiber-level connection from one range of fibers (or ports) to another range of fibers (or ports).

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| id | feature_id | No | Unique ID |
| in_feature | feature_id | No | Cable segment or equipment connection originates at |
| in_side | string | No | Side of feature where connection sits ('in' or 'out') |
| in_low | integer | No | First fiber or port in range |
| in_high | integer | No | Last fiber or port in range |
| out_feature | feature_id | No | Cable segment or equipment runs to |
| out_side | string | No | Side of feature where connection sits ('in' or 'out') |
| out_low | integer | No | First fiber or port in range |
| out_high | integer | No | Last fiber or port in range |
| splice | boolean | No | True if in and out feature are both cable segments |
| housing | feature_id | No | Equipment or structure that is self's direct container |
| root_housing | feature_id | Yes | Structure in which self is located |
| location | point | Yes | Location of self's housing in geographic world |

Notes:

- If root_housing and location are supplied they must match those of housing

- Direction of the connection is not important (has no meaning)

## Line of Count

The line of count and line of count section features are used to model logical assignment of name and status to connected strands (fibers or pairs) in the network. The line of count feature holds the name and status of the assignment and an association to the originating feature for the assignment.

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| Id | feature_id | No | Unique ID |
| origin | feature_id | No | Cable segment or equipment line of count originates at. For equipment, this should be qualified with the side of the equipment the line of count originates at. |
| low_physical | integer | No | First strand assignment applies to. |
| high_physical | integer | No | Last strand assignment applies to. |
| low_logical | integer | No | Start of logical range |
| high_logical | integer | No | End of logical range |
| name | string | No | Assignment name |
| status | string | No | Assignment status |
| physical | boolean | No | Indicates that only status of assignment is to be shown. |
| route | linestring | Yes | Aggregation of line of count section geometry. Will be calculated if not present. |

## Line of Count Section

The line of count section feature relates line of count assignment to a cable segment or equipment ports. It describes how strands at the origin map to strands at the container, which along with the parent line of count feature, is used to calculate the assignment to strands at the container.

| Field | Type | Optional | Comments |
| --- | --- | --- | --- |
| Id | feature_id | No | Unique ID |
| line_of_count | feature_id | No | Line of count feature this is a section for |
| container | feature_id | No | Cable segment or equipment the section applies to. For equipment, this should be qualified with the side of the equipment the section belongs to. |
| mapping | string | No | JSON string containing mapping information. See below for details. |
| route | linestring | Yes | Path of section. Will be calculated from container geometry if not present. |

Notes:

- Each line of count section has a mapping field that describes how each strand at the origin maps to strands on the section’s container. The mapping is a summary of the physical connectivity from the origin of the line of count to the container, and is used to calculate the actual assignment of strands at the container. For example, the mapping:

{"1": [13], "2": [14,15], "3": [16], "4": [17] }

Indicates that the strand 1 at the origin traces to the strand 13 at the container. Strand 2 at the origin traces to strands 14 and 15 at the container (since the trace passed though a splitter).

# Field MetaData

A field metadata file provides information about each attribute of a feature type. Its structure is as follows:

| Field | Type | Comments |
| --- | --- | --- |
| name | string | Field name |
| type | string | Data type of field (see below) |
| unit | string | Unit in which value is represented e.g. 'mm' |

The following data types are supported:

| Data Type | DB Type | Comments |
| --- | --- | --- |
| id | string | Feature ID |
| id_set | string | List of feature IDs, joined with ';' |
| string | string | String (optionally with max length) |
| integer | 32-bit integer | Integer number |
| double | 64-bit float | Floating point number |
| numeric | fixed point | Fixed point value (with precision and dp) |
| boolean | boolean | True/false |
| date | date | Date value |
| timestamp | timestamp | Date and time value |
| link | string | URL string |
| image | base64 string | Image as base64-encode string, with type prefix |
| file | base64 string | File as base64-encode string, with type prefix |

Field metadata files are optional. Their main purpose is to permit unit conversion when loading data.

# Package Metadata

The package metadata file provides global properties of the package. Its structure is:

| Field | Type | Comments |
| --- | --- | --- |
| property | string | Property name (see below) |
| value | <varies> | Property value (see below) |

The following properties are supported:

| Property | Value | Default | Comments |
| --- | --- | --- | --- |
| format | string | 'cdif' | The package format |
| coord_system | integer | 4326 | EPSG code for the map projection in which coordinates are represented |
| boundary | EWKB string | - | Polygon geometry defining extent of data in package (in coord_system) |

The package metadata file and all its properties are optional.

Copyright © 2021, IQGeo UK Ltd 2014 - 2023. All Rights Reserved. You may not reproduce this document in whole or in part without permission in writing from IQGeo at the following address:

IQGeo

Nine Hills Road

Cambridge, CB2 1GE

United Kingdom

WWW: https://www.iqgeo.com/

All contents of this document are subject to change without notice and do not represent a commitment on the part of IQGeo. Reasonable effort is made to ensure the accuracy of the information contained in the document. However, due to on-going product improvements and revisions, IQGeo and its subsidiaries do not warrant the accuracy of this information and cannot accept responsibility for errors or omissions that may be contained in this document.

Information in this document is provided in connection with IQGeo products. No license, express or implied to any intellectual property rights is granted by this document.

IQGeo encourages all users of its products to procure all necessary intellectual property licenses required to implement any concepts or applications and does not condone or encourage any intellectual property infringement and disclaims any responsibility related thereto. These intellectual property licenses may differ from country to country and it is the responsibility of those who develop the concepts or applications to be aware of and comply with different national license requirements.

Windows® is a registered trademark of Microsoft Corporation in the United States and/or other countries. The other names of actual companies and products mentioned herein are the trademarks of their respective owners.